# 3D Milk Industry Simulation — Research & Plan

Oct 10, 2026 · @Jojo

## Goal & scope

Build a browser-based 3D simulation in Three.js of a UHT milk plant that runs all 10 stages, from standardization to ambient distribution, with animated equipment and live process data, over a 12-week term.

What the viewer sees and does

- Follows milk from the raw-milk silo to a delivery truck through one connected 3D plant.

- Reads live values at each stage: temperature, pressure, flow rate, tank level, packs and pallets counted.

- Changes a setting (UHT temperature, filler speed, QA result) and watches the plant react.

In scope

- One product: whole UHT milk (3.5% fat) in generic 1-litre aseptic brick cartons.

- Simplified physics: mass balance, temperature profile and timing that are believable, not engineering-grade.

- A time-speed control, so a 7-day quarantine and weeks of warehousing pass in minutes.

- Six fault scenarios with visible consequences, listed under Simulation model.

Out of scope (stretch goals at most)

- Fluid dynamics (CFD), detailed heat-transfer equations, fouling chemistry.

- Farm collection and raw-milk intake testing; the model starts at the raw-milk silo.

- Cleaning-in-place (CIP) cycles beyond a status light.

- Real brands, VR and mobile layouts.

Assumptions: one developer, desktop browser (Chrome or Edge).

## The UHT line at a glance

The 10 stages form one line with a single hand-off: at the aseptic tank, flowing milk becomes packs you can count, track and hold.

process flow · 10 stages, one hand-off

UHT is the critical control point: when the sterilizing effect drops, milk is diverted before it reaches the tank.

3D preview: UHT Milk Plant Preview runs this flow in the browser with grey-box models, example data, speed controls and three fault scenarios.

sim defaults and source ranges · Tetra Pak handbook, Regulation 853/2004, GEA

The sim uses the dots as set points and colours each pipe from them; the separator step sits in the upstream milk-treatment line, before the milk is cooled and stored for UHT.

## Stage research 1–5: the liquid line

Stages 1–5 are one continuous flow of liquid, so the simulation tracks them in litres per hour, temperature and pressure.

### 1. Standardization & Formulation

A separator splits warm milk into skim and cream, and in-line control remixes just enough cream to hit the target fat (3.5% here).

- Real process: Raw milk waits in insulated silo tanks of 30,000–250,000 L with side agitators (Tanks). At 55–65 °C a disc-stack separator gives skim milk (about 0.05% fat) and cream (35–40%). Flow and density transmitters drive a valve that remixes cream, holding fat within 0.015% (Separators and standardization).

- Formulation: stabilizers, vitamins or flavours go in through batch tanks of 250–10,000 kg or in-line mixers; high-shear mixers handle hard powders (Mixers).

- 3D: silo with level gauge, separator with a spinning bowl, standardization skid (flow meters, valves), mixing tank with a turning agitator.

- Simulate: fat % by mass balance, surplus cream (L/h), silo and mix-tank levels, recipe per batch.

### 2. Preheating & Deaeration

Heat exchangers warm the cold feed to about 75–80 °C, then a vacuum vessel strips out the air.

- Real process: Milk leaves the feed tank at about 4 °C, passes a balance tank and feed pump, and is preheated to about 75 °C in indirect plants, using heat recovered from outgoing milk, or about 80 °C in direct plants (Long-life milk). In the deaerator, vacuum makes the milk boil 4–5 °C below its preheat temperature; a vacuum pump removes the air and a built-in condenser returns the vapour (Deaerators).

- Why it matters: air fouls heating surfaces and skews flow measurement (Deaerators).

- 3D: balance tank, feed pump, plate or tubular heat exchanger, vacuum vessel with condenser on top and a vacuum pump beside it.

- Simulate: outlet temperature from the set point, the 4–5 °C drop in the vessel, air content falling (a sim value), heat recovered in kW.

### 3. Homogenization

A high-pressure piston pump forces milk through a gap of about 0.1 mm, so fat globules shrink below 1 µm and no cream layer forms during months on the shelf.

- Real process: UHT lines homogenize at 180–250 bar (Long-life milk). The pump has 3–5 pistons; the first stage breaks the globules (about 3.5 µm down to under 1 µm), and the second adds about 30 bar back-pressure to break clusters. It runs at 55–80 °C, and every 40 bar of pressure drop adds 1 °C (Homogenizers).

- Position note: your stage order puts homogenization before UHT, as indirect plants do. Direct plants with flash cooling use an aseptic homogenizer after the flash vessel instead (Homogenizers).

- 3D: box-shaped machine with three pistons driven by a crank, two pressure gauges, motor with belt drive.

- Simulate: stage 1 and stage 2 pressures, temperature rise, globule size, a creaming-risk score, power draw (handbook example: 68 kW for 10,000 L/h at 200 bar).

### 4. UHT Treatment

Steam heats the milk to about 140 °C, a holding tube keeps it there for about 4 s, and any drop below the set point diverts the flow to reject.

- Real process: milk needs 135–150 °C for a few seconds, in practice 137–142 °C for 4–6 s (Long-life milk); EU law requires at least 135 °C (Regulation 853/2004, Annex III §IX). Direct plants inject steam through a ring nozzle to 140–150 °C, under pressure so it cannot boil. The target is a 9-log kill of heat-resistant spores (B* above 1) and an F0 of 5–6 min (UHT milk module).

- Safety logic: temperature is recorded at the holding-tube outlet (FDA aseptic guide). If it drops, product goes to a reject tank, and the plant is flushed, cleaned and re-sterilized before restart. Before each run, hot water at 125 °C or more circulates for at least 30 min (Long-life milk).

- 3D: steam injector head, coiled holding tube, flow-diversion valve with a green/red light, steam pipe from the boiler house.

- Simulate: holding time = tube volume ÷ flow, live F0 and B*, divert events, plant capacity (direct systems run 2,000–45,000 L/h).

### 5. Flash Cooling

A vacuum vessel drops the milk from about 140 °C to about 80 °C almost instantly and boils off the same amount of water the steam added.

- Real process: the vacuum is set so the vapour flashed off equals the steam injected, and the milk falls back to its preheat temperature, about 80 °C. A pump then feeds the aseptic homogenizer, and a tubular cooler brings the milk to about 20 °C for the aseptic tank or filler (Long-life milk).

- 3D: tall vessel with a conical bottom, vapour pipe to a condenser, extraction pump, tubular cooler.

- Simulate: water removed = steam added (solids stay constant), temperature steps to 80 °C and then 20 °C, condenser load.

## Stage research 6–10: from sterile tank to truck

After the aseptic tank, milk becomes countable units (packs, trays, pallets), so the simulation switches from flow rates to discrete events.

### 6. Aseptic Storage (Buffer)

A sterile tank sits between the UHT plant and the fillers, so either side can stop or slow down without stopping the other.

- Real process: the tank balances supply to one or more fillers. The UHT plant is sized slightly larger, so the level builds up and the fillers keep running through its 30-minute intermediate clean; indirect plants can also run at 50–100% of capacity (Long-life milk). A Tetra Alsafe tank holds 7,000–50,000 L, is steam-sterilized at 125 °C or more, and keeps sterile air above the milk at up to 3.0 bar; a load cell in one leg weighs the contents.

- Rule: sterile-air overpressure must be kept during operation, and a failed air filter counts as a process deviation (FDA aseptic guide).

- 3D: vertical tank on three legs, valve cluster, sterile-air filter housing, floating level label.

- Simulate: level changes by (inflow − outflow) each tick; high level throttles the UHT plant to 50%, low level pauses the fillers; headspace pressure.

### 7. Aseptic Packaging

The filler sterilizes the carton material, forms it into a tube, fills it with sterile milk and seals each pack below the liquid level, so no air or germs get in.

- Real process: material comes off a reel and a plastic strip is sealed to one edge. The web runs through 30–35% hydrogen peroxide, then hot air at 60–125 °C removes the residue. It is formed into a tube, sealed lengthwise, and sealed across below the product level (ICPE aseptic packaging notes); the tube is then cut and folded into bricks.

- Example machine: the Tetra Pak A3/Flex fills up to 8,000 packages per hour in 500–2,000 ml formats, with a peroxide bath and a sterile-air zone under closed-loop control. US rules cap peroxide at 35% and residues at 0.5 ppm (UHT milk module).

- 3D: filler cabinet with a glass aseptic zone, paper reel, vertical tube, moving sealing jaws, outfeed conveyor of cartons.

- Simulate: packs per hour, random stops (time between failures, repair time), rejected packs, batch and date code on every pack.

### 8. Secondary Packaging & Palletization

Conveyors group packs into shrink-wrapped trays, and a palletizer stacks the trays into labelled pallets, the unit tracked from here on.

- Real process: downstream machines add straws to small portion packs, at up to 40,000 packs per hour (Straw applicators), and shrink-wrap trays. The Tetra Pak Tray Shrink 30 takes 40 tray sizes and 40 µm film, and speeds up by as much as 30% to clear a queue after a stop.

- Pallet label: each pallet gets an 18-digit SSCC; the GS1 label also shows product GTIN, quantity, batch number and best-before date (GS1 UK).

- 3D: conveyors with an accumulation table, tray-shrink tunnel, robot-arm palletizer, stretch wrapper with a rotating arm, label printer.

- Simulate: 12 packs per tray and 84 trays per pallet (sim assumptions, about 1,000 L), conveyor queues, catch-up speed after stops, one SSCC per pallet.

### 9. Quarantine & Quality Assurance Testing

Every batch stays on hold until its sample packs pass incubation, so a pallet can be finished but not yet sellable.

- Rule: EU law requires UHT products to stay microbiologically stable after 15 days at 30 °C or 7 days at 55 °C in closed containers (Regulation 853/2004, Annex III §IX). US rules do not make incubation mandatory, but inspectors check sampling, incubation conditions and lot acceptance criteria when it is used (FDA aseptic guide).

- Checks after incubation: colony count, pH and acidity change, smell and taste, ethanol stability; in one study a resazurin test caught every spoiled sample after 3 days at 30 °C (Journal of Dairy Research, 1990). The typical microbial failure rate is about 0.02% (UHT milk module).

- 3D: quarantine zone with yellow floor lines and a status lamp on each pallet, QA lab with incubators and sample packs.

- Simulate: samples per batch, incubation timer (time-compressed), pass or fail drawn from a failure rate, status HOLD → RELEASED or BLOCKED.

### 10. Warehousing & Ambient Distribution

Released pallets are stored and shipped at room temperature with no cold chain, and are picked by best-before date.

- Real process: UHT milk keeps six to nine months at 20–30 °C without refrigeration (GEA). Its shelf life ends through physical and enzyme changes such as creaming and gelling, not microbes; the handbook's examples are 3 months for local sale and 9 months for export (Long-life milk).

- 3D: pallet racks, forklifts or AGVs on floor paths, loading dock, truck that leaves when full.

- Simulate: stock by batch and best-before date, first-expired-first-out picking, random daily orders, truck dispatch, order fill rate.

## Plant layout & 3D asset list

Lay the plant out as one U-shaped flow through seven zones, block everything out with simple shapes first, and spend real modelling time only on four hero machines: separator, homogenizer, filler and palletizer.

floor plan sketch · 7 zones, not to scale

The U shape lets the guided tour follow the product in one loop and keeps quarantine right next to the warehouse.

| Asset | Stage | Build with | Animation | Priority |
| --- | --- | --- | --- | --- |
| Raw-milk silos (2) | 1 | Code: cylinders | Level rises and falls | Must |
| Separator | 1 | Blender (hero) | Bowl spins | Must |
| Mixing tank with agitator | 1 | Code: cylinder + shaft | Agitator turns, level | Must |
| Standardization skid | 1 | Code: pipes, small boxes | Valve opening % | Should |
| Balance tank, pumps | 2 | Code | Impellers spin | Should |
| Plate heat exchanger | 2 | Code: instanced plates | Glows by temperature | Must |
| Deaerator with condenser | 2 | Code: cylinders | Bubbles rise | Must |
| Homogenizer | 3 | Blender (hero) | Three pistons pump | Must |
| Steam injector, holding tube | 4 | Code: TubeGeometry helix | Heat colour, steam puffs | Must |
| Flow-diversion valve, reject tank | 4 | Code | Valve flips, red light | Must |
| Flash vessel, tubular cooler | 5 | Code | Vapour plume | Must |
| Aseptic tank | 6 | Code: cylinder on three legs | Level and pressure | Must |
| Aseptic fillers (2) | 7 | Blender (hero) | Reel turns, jaws seal, packs exit | Must |
| 1 L carton, tray, pallet | 7–10 | Code: boxes + generic texture | InstancedMesh movement | Must |
| Conveyors, accumulation table | 7–8 | Kenney Conveyor Kit (CC0, 60+ models) | Belt texture scrolls | Must |
| Tray-shrink tunnel | 8 | Code: box tunnel | Trays pass through | Should |
| Palletizer robot | 8 | Blender (hero) | Pick-and-place loop | Should |
| Stretch wrapper | 8 | Code: turntable + mast | Turntable rotates | Could |
| Quarantine zone, QA lab | 9 | Floor decals, boxes | Status lamps | Must |
| Racks, forklifts, trucks | 10 | Instanced racks, CC0 models | Path following | Should |
| Pipes | All | TubeGeometry along curves | Flowing texture, temperature colour | Must |
| Boiler house, CIP station | 4 | Code: boxes | Status light | Could |

Keep cartons and trays brand-free: a plain white brick with an invented logo. Export Blender models as GLB and load them with GLTFLoader.

## Simulation model

Run two coupled models: a fixed-step continuous model for stages 1–6 (litres, °C, bar) and an event-driven discrete model for stages 7–10 (packs, trays, pallets, batches), joined at the aseptic tank.

How time works

- One sim clock with speed presets from ×1 to ×10,000: ×1 to watch the 4-second holding tube, ×10,000 to pass a 7-day quarantine in about a minute.

- The continuous model steps every simulated second, several steps per frame; above ×1,000 it steps every 10–60 s, since tank levels change slowly.

- The discrete model keeps an event queue sorted by time (filler fault, repair done, pallet full, incubation over, truck arrives) and runs every event due before each new frame.

- The model never touches Three.js. Each frame the 3D view reads a state snapshot and animates a sample of visual cartons, not every pack.

Core formulas

Cream share to remix for 3.5% milk from 0.05% skim and 40% cream:

x_{\text{cream}} = \frac{f_{\text{target}} - f_{\text{skim}}}{f_{\text{cream}} - f_{\text{skim}}} = \frac{3.5 - 0.05}{40 - 0.05} \approx 0.086

Tank level per step, and holding time from tube volume and flow:

V_{t+\Delta t} = V_t + (Q_{\text{in}} - Q_{\text{out}})\,\Delta t \qquad t_{\text{hold}} = \frac{V_{\text{tube}}}{Q}

Sterilizing effect in the holding tube (t in seconds, T in °C), from the F0 and B* reference values in the UHT milk module:

F_0 = \frac{t_{\text{hold}}}{60} \cdot 10^{(T - 121.1)/10} \qquad B^{*} = \frac{t_{\text{hold}}}{10.1} \cdot 10^{(T - 135)/10.5}

At 140 °C for 4 s this gives F0 ≈ 5.2 min and B* ≈ 1.19, inside the targets. Divert when B* falls below 1, which for a 4 s hold means below about 139.2 °C.

Filler efficiency, as defined by OEE.com:

\text{OEE} = \text{Availability} \times \text{Performance} \times \text{Quality}

Default settings (temperatures are in the chart under The UHT line at a glance)

| Setting | Default | Basis |
| --- | --- | --- |
| UHT plant capacity | 16,000 L/h | Direct plants run 2,000–45,000 L/h (Long-life milk) |
| Fillers | 2 × 7,500 packs/h, 1 L | Below the 8,000/h maximum of the A3/Flex |
| Homogenizer | 200 bar + 30 bar | 180–250 bar; second stage about 30 bar (Homogenizers) |
| Holding time | 4 s | Practical range 4–6 s (Long-life milk) |
| Aseptic tank | 20,000 L, starts 50% full | Sizes run 7,000–50,000 L (Tetra Alsafe) |
| Cleaning | Full CIP 80 min; intermediate clean 30 min | CIP takes 70–90 min, AIC 30 min (Long-life milk) |
| Batch | One filler shift of 8 h (60,000 packs) | Sim assumption |
| QA samples | 1 pack per filler every 30 min, plus 2 after each stop | Sim assumption |
| Quarantine | 7 days (55 °C option) or 15 days (30 °C) | Regulation 853/2004 |
| Microbial failure rate | 0.02% of packs | UHT milk module |
| Pack, tray, pallet | 1 L; 12 packs; 84 trays | Sim assumptions |
| Shelf life | 6 months | Six to nine months (GEA) |
| Truck | 33 pallets | Standard 13.6 m trailer (Eurowag) |

KPIs on screen

- Liquid line: UHT throughput (L/h), F0 and B*, litres diverted, aseptic tank level (%).

- Filling and packing: packs per hour and OEE per filler, trays and pallets made.

- QA and warehouse: batches on hold, released and blocked; pallets in stock; days of cover; order fill rate; trucks shipped.

Fault scenarios

| Scenario | Trigger | Plant reaction | What the viewer sees |
| --- | --- | --- | --- |
| UHT temperature drop | Steam supply dips, B* < 1 | Divert to reject; flush, clean, re-sterilize 30 min at 125 °C or more; fillers drain the tank | Valve light turns red, reject tank fills, tank level falls |
| Filler breakdown | Random, from time between failures | Tank level rises; at high level the UHT plant drops to 50% | Filler lamp red, carton flow stops, level climbs |
| Sterile-air filter failure | Manual trigger | Process deviation; tank contents put on hold | Tank lamp amber, batch marked for investigation |
| Failed incubation | Sample fails (chance rises after a filter failure) | Batch blocked, pallets pulled for destruction | Pallet lamps in quarantine turn red |
| Homogenizer pressure loss | Manual trigger | Creaming-risk score rises; QA sensory check fails days later | Gauge needle drops, delayed QA failure |
| Demand spike or late truck | Manual or random | Stock runs down, or the dock backs up | Rack fill colours change, trucks queue |

## Tech stack & architecture

Use three.js with Vite and TypeScript, keep the simulation core free of 3D code, and let the 3D view and the UI read one shared state snapshot every frame.

| Need | Choice | Why |
| --- | --- | --- |
| 3D engine | three.js r186, npm package three | Latest release, September 2026 |
| Build and dev server | Vite | The three.js manual's recommended setup: npm install --save three, npm install --save-dev vite, npx vite |
| Language | TypeScript (plain JavaScript also works) | Types catch unit mix-ups such as L/h versus L/s |
| Camera | OrbitControls + a scripted tour | Free look plus guided stage-by-stage flights |
| Labels | CSS2DRenderer | HTML labels stay sharp and clickable |
| Cartons, trays, pallets | InstancedMesh | Thousands of copies in one draw call |
| Pipes | TubeGeometry along curves | One mesh per pipe run, recoloured by temperature |
| Models | Blender → GLB → GLTFLoader | Standard web 3D format |
| Debug sliders | lil-gui | Floating panel to change set points live |
| Live charts | Chart.js | Trend lines for temperature, tank level, OEE |
| Tests | Vitest | Vite-native; tests for F0, mass balance and the event queue |
| Hosting | GitHub Pages or Vercel | One link to share with your teacher |

architecture · src/ folders and data flow per frame

Because the core is plain TypeScript with no 3D code, Vitest can test it without a browser.

Folder structure

src/
  main.ts            boot: load config, start the frame loop
  sim/
    clock.ts         sim time, speed, pause
    eventQueue.ts    priority queue of timed events
    liquidLine.ts    stages 1–6: flows, temperatures, levels, F0/B*, divert
    filler.ts        stage 7: one state machine per filler
    packaging.ts     stage 8: trays, pallets, SSCC numbers
    quality.ts       stage 9: batches, samples, incubation
    warehouse.ts     stage 10: stock, FEFO picking, trucks
    scenarios.ts     fault injection
  scene/
    world.ts         renderer, lights, floor, cameras
    equipment/       one class per machine: build() + update(snapshot)
    pipes.ts         tubes, flow texture, temperature colour
    tokens.ts        InstancedMesh cartons, trays, pallets
  ui/
    hud.ts  panel.ts  charts.ts  tour.ts
  data/
    plant.json       the default settings from the Simulation model section

Every frame

- Turn UI input into commands: speed, set points, scenario triggers.

- Advance the sim: continuous steps, then every event that is due.

- Build a plain-object snapshot of the plant state.

- Update the 3D scene from the snapshot: machines, pipe colours, carton tokens.

- Update the HUD and charts, throttled to 4 times a second.

- Render.

Performance budget

- 60 fps on a mid-range laptop.

- One InstancedMesh per carton, tray and pallet type; merge static pipes and floors.

- One shadow-casting light; bake ambient occlusion into the hero models in Blender.

## User interface & interactions

Use one screen: the 3D plant fills the window, a top bar controls time, a left rail jumps between the 10 stages, a right panel explains the selected machine, and a bottom strip shows the KPIs.

screen wireframe · homogenizer selected, stage 9 on hold

The stage rail and the info panel stay in sync: clicking a machine in 3D selects its stage, and clicking a stage flies the camera to it.

Controls

- Top bar: day and time, pause, speed (×1, ×10, ×100, ×1,000, ×10,000), jump to next event, scenario menu.

- Left rail: stages 1–10, each with a status lamp; a click flies the camera there.

- Right panel: what the machine does (from the stage research), live values, a small trend chart, and sliders for set points that make sense (UHT temperature, filler speed).

- Bottom strip: the KPIs listed under Simulation model.

- In the scene: a floating label with one key value per machine; pipes coloured by temperature with a legend; click a pallet to see its SSCC, batch and status.

Modes

- Free explore: orbit, pan and zoom; double-click a machine to focus on it.

- Guided tour: the camera visits stages 1–10 in order with a two-line caption each, about 3 minutes in total.

- Scenario: pick a fault, watch the plant react, then read a short log of what happened and why.

Colour rules

- Pipes: blue for cold milk, through yellow, to red at UHT temperature.

- Lamps: green for running, amber for on hold or warning, red for stopped or blocked.

- The same three lamp colours apply everywhere, including pallets in quarantine.

## Development roadmap

Build in six phases over 12 weeks: grey-box the whole plant first, make the liquid line work, then the packaging and logistics, and only then spend time on detailed models and polish.

12-week plan · 6 phases, 3 review gates

Gate 2 is the safety net: if the run from silo to truck does not work by week 9, cut Could items instead of stretching the schedule.

Deliverables to tick off

- ☐ Week 1: Vite + three.js project runs; plant.json holds the default settings

- ☐ Week 3 (gate 1): grey-box plant with all seven zones, pipes and camera fly-to

- ☐ Week 6: stages 1–6 live (levels, temperatures, F0 and B*, divert); formula tests pass

- ☐ Week 9 (gate 2): stages 7–10 live (packs, pallets, quarantine, warehouse, trucks); scope frozen

- ☐ Week 11: hero models, six fault scenarios, guided tour

- ☐ Week 12 (gate 3): 60 fps check, demo video, report and presentation

If the term runs past 12 weeks, the stretch list is: animated CIP cleaning, a steam and energy model, a switch between direct and indirect UHT, and a mobile layout.

## Risks & simplifications

The biggest risk is scope: ten stages plus detailed 3D is a lot for one person, so every stage gets a working grey-box version before any one stage gets polish.

| Risk | Effect | Mitigation |
| --- | --- | --- |
| Scope too big for one person | Stages unfinished at the deadline | Grey-box all ten stages by week 3; Must/Should/Could priorities; scope freeze at week 9 |
| Too many objects on screen | Frame rate drops with thousands of packs and pallets | InstancedMesh, merged static meshes, visual tokens instead of one mesh per pack |
| Time scales from 4 seconds to 6 months | Viewers miss what happens | Speed presets, jump to next event, event log |
| Stage order mixes plant types | A reviewer questions where the homogenizer sits | Keep the course order and explain it in an "About this model" panel |
| 3D modelling eats the schedule | Simulation logic slips | Only four hero models; primitives and CC0 kits for the rest |
| Wrong numbers in formulas | Misleading KPIs | Unit tests against hand calculations, such as F0 ≈ 5.2 min at 140 °C for 4 s |

Accepted simplifications

- Milk moves as plug flow, with no pressure drop, viscosity effects or fouling.

- Temperatures change in steps with a short lag, not through heat-transfer equations.

- One product and one pack size.

- Cleaning is a timed machine state, not liquid moving through the pipes.

- Failures follow simple random rates, not real plant data.

## Sources

Process and regulation

- Tetra Pak Dairy Processing Handbook: Long-life milk

- Tetra Pak Dairy Processing Handbook: Centrifugal separators and milk standardization

- Tetra Pak Dairy Processing Handbook: Deaerators

- Tetra Pak Dairy Processing Handbook: Homogenizers

- Tetra Pak Dairy Processing Handbook: Tanks

- Tetra Pak Dairy Processing Handbook: Mixers

- UHT milk and milk products (INFLIBNET e-PG Pathshala module)

- Regulation (EC) No 853/2004, Annex III, Section IX

- FDA: Aseptic Processing and Packaging for the Food Industry

- Reinheimer & Demkow (1990), Comparison of rapid tests for assessing UHT milk sterility, Journal of Dairy Research

- GEA: Shelf-stable milk

Equipment and logistics

- ICPE: Aseptic packaging system (PDF)

- Tetra Alsafe aseptic tank product description (PDF)

- Tetra Pak A3/Flex filling machine

- Tetra Pak straw applicators

- Tetra Pak Tray Shrink 30

- GS1 UK: Logistics labels and SSCCs

- Eurowag: How many euro pallets fit on a truck

- OEE.com: Calculating OEE

Tools

- three.js releases and three.js manual: Installation

- three.js docs: InstancedMesh, TubeGeometry, CSS2DRenderer, GLTFLoader, OrbitControls

- Kenney Conveyor Kit, lil-gui, Vitest